CytoFLEX Nano

V02

Relevant and important information for publication of flow cytometric data

Remember to acknowledge the FACS Core Facility, Aarhus University, in your publications and presentations (oral presentations and posters), according to our guidelines.

Send your papers to facs@au.dk for review and we will give you feedback within 2 workdays.

The Instrument is funded by the Carlsberg Foundation. Therefore, you MUST include the following in your acknowledgement: "The work presented here is supported by the Carlsberg Foundation, grant CF23-1020".

Methods and materials:

Instrument

- CytoFLEX nano small particle flow cytometer equipped with four lasers (405 nm, 488 nm, 561 nm, and 638 nm),6 fluorescence detectors, 1 forward scatter detector, and 5 side scatter detectors (Beckman Coulter, Souzhou, China).

Software

- Software: CytExpert nano (v. 1.1.0.6, Beckman Coulter, Souzhou, China).

- Other analysis software programs used: e.g. FlowJo or FCS Express (incl. version and company).

Experimental

- Antibodies (clone, isotype, manufacturer, fluorochrome) used and in which filters they were detected.

- Information about filters used during sample preparation, antibody concentrations, and staining conditions (incubation time, temperature, buffers, blocking, etc.). State if antibody titration was performed.

- Controls included: E.g. compensation controls, biological controls, FMO controls, buffer only, buffer + reagent, dilution controls, unstained EVs, detergent control, single stains, isotype controls, and procedural controls.

- For compensation, state if you used beads or other particles.

- Stop conditions: E.g. time, number of events in a specific gate, or a sample volume.

Abbreviations:

| Forward Scatter | Side scatter | Violet | Blue | Yellow | Red |
| --- | --- | --- | --- | --- | --- |
| FSC | SSC | V | B | Y | R |

All lasers are colinear.

| Laser | 405 nm | 488 nm | 561 nm | 638 nm |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Laser power | 120 mW | 50 mW | 35 mW | 100 mW |  |  |
| Filters | 447/60 | 531/46 | 595/50 | 670/30 | 710/46 | 792/64 |
| Scatters | VFSC 405/10 | VSSC1 405/10* | VSSC2 405/9** | BSSC 488/10 | YSSC 561/10 | RSSC 638/10 |

*For detection of particles between 40# and150 nm

**For detection of particles between 80 and 1000 nm.

#Based on polystyrene beads.